# Activity C19: Determine the Equilibrium Constant, Kc, of a Reaction (Colorimeter)

| Concept | #### DataStudio | ScienceWorkshop (Mac) | ScienceWorkshop (Win) |
| --- | --- | --- | --- |
| Rates of reaction | C19 Kc Constant.DS | See appendix. | See appendix. |

| ## Equipment Needed | ## Qty | ## Chemicals and Consumables | ## Qty |
| --- | --- | --- | --- |
| Colorimeter (CI-6747) | 1 | ## Iron nitrate, Fe(NO3)3, 0.200 molar | 20 mL |
| Beaker, 100-mL | 3 | ## Iron nitrate, Fe(NO3)3, 0.002molar | 30 mL |
| Cuvette (w/sensor) | 1 | Label | 5 |
| Pipette, 10 mL, with pipette bulb or pump | 4 | Potassium thiocyanate, KSCN, 0.002 M | 25 mL |
| Stirring rod | 1 | Tissue | 4 |
| Test tube, 20 by 150 mm | 5 | ## Water, distilled | ## 30 mL |
| Test tube rack | 1 |  |  |
| Thermometer (SE-9084) | 1 |  |  |
| Protective gear | PS |  |  |

## What Do You Think?

In some chemical reactions the amounts of products formed can be calculated when the amounts of reactants are known, assuming that the reaction goes to completion so that one of the reactants is used up. However, many reactions to not go to completion. Instead, they reach an equilibrium state in which both reactants and products are present. The ratio of the concentrations of reactants to products at a given temperature is the equilibrium constant, Kc. The equilibrium constant allows the calculation of amounts of substances present at equilibrium. How can a Colorimeter be used to determine the equilibrium constant of a chemical reaction?

Take time to answer the ‘What Do You Think?’ question(s) in the Lab Report section.

## Background

When iron ions (Fe3+) and thiocyanate ions (SCN-) are combined, equilibrium is established between these two ions and the thiocyanoiron ion (FeSCN2+).

Fe3+(aq) + SCN−(aq)ÛFeSCN2+(aq)

In order to calculate Kc for the reaction, you need to know the concentrations of all ions at equilibrium: [FeSCN2+]eq, [SCN-]eq, and [Fe3+]eq. (Note: The square brackets [] around a reactant or product indicates its concentration in moles per liter.)

The FeSCN2+ ion in solution produces a red color. The red solutions absorb blue light very well. A Colorimeter can measure the amount of blue light absorbed by the colored solutions. By comparing the absorbance, Aeq, of different equilibrium systems with unknown concentrations of thiocyanoiron ions to the absorbance, Astd, of a standard solution, you can use Beer’s Law to determine the concentration of the thiocyanoiron ion [FeSCN2+]eq. Beer’s Law describes the linear relationship between the absorbance and concentration for a solution. The standard solution has a known FeSCN2+ concentration.

Assuming that the concentration of the thiocyanoiron ion (FeSCN2+) and the absorbance are directly related (Beer's Law), the concentration of FeSCN2+ for any of the equilibrium systems can be found by:

[FeSCN2+]eq=  AeqAstdx[FeSCN2+]std

Knowing the [FeSCN2+]eq allows you to determine the concentrations of the other two ions at equilibrium. Because the ratio of coefficients in the reaction equation are one-to-one, for each mole of FeSCN2+ ions produced, one less mole of Fe3+ ions remain in the solution.

The [Fe3+] can be determined by:

[Fe3+]eq = [Fe3+]i±[FeSCN2+]eq

Because one mole of SCN- is used up for each mole of FeSCN2+ ions produced, [SCN-]eq can be determined by:

[SCN−]eq = [SCN−]i±[FeSCN2+]eq

You can calculate the value of Kc, the equilibrium constant from the values of [Fe3+]eq, [SCN-]eq, and [FeSCN2+]eq.

| ## SAFETY REMINDERS - Wear protective gear. - Follow directions for using the equipment. - Handle and dispose of all chemicals and solutions properly. |  |  |  |
| --- | --- | --- | --- |

CAUTION: Never pipette by mouth. Always use a pipette bulb or a pipette pump. Be careful when handling any acid or base solutions.

## For You To Do

Prepare four solutions with different concentrations of the three ions (Fe3+, SCN- and FeSCN2+). Also prepare a standard solution of thiocyanoiron (FeSCN2+). Use the Colorimeter to measure the absorbance of each of the four solutions and also the absorbance of the standard solution. Use DataStudio or ScienceWorkshop to record and display the data.

Use the absorbance and concentration of the standard solution to calculate the equilibrium concentrations of the other four solutions. Calculate the equilibrium concentrations of each of the ions. Use these values concentration values and the ratio for the equilibrium constant expression to calculate Kc.

## PART I: Computer Setup

1.	Connect the ScienceWorkshop interface to the computer, turn on the interface, and turn on the computer.

2.	Connect the DIN plug of the Colorimeter cable to Analog Channel A on the interface.

•	The Colorimeter will automatically turn itself on when it is connected to the ScienceWorkshop interface.

3.	Open the file titled as shown;

| #### DataStudio | ScienceWorkshop (Mac) | ScienceWorkshop (Win) |
| --- | --- | --- |
| C19 Kc Constant.DS | See appendix. | See appendix. |

•	The DataStudio file has a Digits display and a Table display of absorbance. Read the Workbook display for more information.

•	See the appendix for instructions on creating a ScienceWorkshop document for this activity.

•	Data recording is set at ten measurements per second (10 Hz). Data recording is also set so that you can manually enter the number of each solution.

## PART II: Sensor Calibration and Equipment Setup

### About the Colorimeter

The Colorimeter analyzes colors of light that pass through a solution. The solution is put into a rectangular container called a cuvette, which is then placed inside the Colorimeter. The measure of the amount of light that passes through a solution is called “transmittance”. Transmittance is a ratio of the intensity of the transmitted light to the intensity of the original light, and is usually expressed as a percentage.

Absorbance is related to transmittance. The light absorbed by a solution depends on the absorbing ability of the solution, the distance traveled by the light through the solution, and the concentration of the solution. The relationship of absorbance to transmittance is:

### Calibration

The general method for calibrating the Colorimeter is as follows:

•	First, calibrate the Colorimeter with a clear cuvette containing distilled water.

•	Second, calibrate the software (either DataStudio or ScienceWorkshop) for one of the four colors of light that can be selected in the Colorimeter. (For this activity you will use the BLUE wavelength.)

Note: The cuvette has two clear sides and two ridged sides.

•	All cuvettes should be wiped clean and dry on the outside with a tissue.

•	Handle cuvettes only by the top edge of the ridged sides.

•	All solutions should be free of bubbles.

•	Always position the cuvette so the light beam will pass through the clear sides.

### Calibrate the Colorimeter

When the Colorimeter comes on, the liquid crystal display (LCD) should say “Please calibrate”.

To calibrate the Colorimeter with a clear cuvette, fill a clean cuvette with distilled water and cap the cuvette. (The clear cuvette is a control or ‘reference’ that accounts for the small amount of light scattered or reflected by the walls of the cuvette.)

On the Colorimeter, press the ‘Select’ button () and the ‘Start/Stop’ button () at the same time.

The Colorimeter’s LCD will say “Insert reference then push SELECT”.

Clear sides aligned with light path.

Place the closed cuvette inside the Colorimeter. Make sure that the clear sides of the cuvette (without ridges) are lined up with the light path in the Colorimeter. Close the lid on the Colorimeter.

On the Colorimeter, press the ‘Select’ button.

The Colorimeter will automatically calibrate itself for all four wavelengths assuming that the light passing through the clear cuvette represents “100% Transmittance”. (The automatic calibration takes only a few seconds.)

The Colorimeter’s LCD will say “CAL done, push SELECT or START”.

### Calibrate the Software

Follow these steps to calibrate the software for one of the four colors of light:

1.	Leave the cuvette with distilled water inside the Colorimeter.

2.	In the Experiment Setup window, double-click the Colorimeter icon.

•	In DataStudio, the Sensor Properties window will open. Click the ‘Calibration’ tab. In ScienceWorkshop, the Sensor Setup window will open.

3.	Select the color of light. Use BLUE light for this activity.

Press the ‘Select’ button until the LCD shows “Blue 460 nm”.

4.	Calibrate the software.

•	First, press the ‘Start/Stop’ button () to start the Colorimeter. (The LCD shows the color and wavelength, the percent transmittance, and “RUN”.)

•	Second, check the voltage under ‘Current Reading’ in DataStudio or next to ‘Cur Value:’ in ScienceWorkshop.

•	Third, when the voltage stabilizes, click the ‘Take Reading’ button under ‘High Point’ in DataStudio or the ‘Read’ button in the row for ‘High Value:’ in ScienceWorkshop.

•	Fourth, press the ‘Start/Stop’ button to stop the Colorimeter. (The LCD changes to “STOP”.)

5.	Click ‘OK’ to return to the Experiment Setup window.

•	The software is now calibrated for the Colorimeter.

### Equipment Setup

SAFETY CAUTION! Be sure to wear protective gear (splash shield, gloves, apron or lab coat). The iron nitrate solutions used in this activity were prepared in 1 molar nitric acid. Handle these and all chemicals with extreme care. Use a pipette with a pipette bulb or a pipette pump to pipette the chemicals. Do Not Pipette By Mouth!

### Prepare the Solutions

•	For this part you need the following: test tubes (4), labels (4), 100-mL beakers (3), pipettes (4), stir rod, 25 mL of 0.002 M iron nitrate (Fe(NO3)3), 30 mL of 0.002 M potassium thiocyanate (KSCN), 30 mL of distilled water.

1.	Put labels on four clean, dry test tubes and mark them #1 through #4.

2.	Put 30 mL of 0.002 iron nitrate (Fe(NO3)3 into a beaker. Use a pipette to put 5.0 mL of the solution into each of the four test tubes.

3.	Put 30 mL of 0.002 potassium thiocyanate (KSCN) into a second beaker. Use a pipette to put 2 mL into Test Tube #1, 3 mL into Test Tube #2, 4 mL into Test Tube #3, and 5 mL into Test Tube #4.

4.	Put about 25 mL of distilled water into a third beaker. Use a pipette to put 3 mL into Test Tube #1, 2 mL into Test Tube #2, and 1 mL into Test Tube #3. (Note: Don’t add any distilled water to Test Tube #4.)

•	The summary of components in the test tubes is as follows:

| Test Tube | Fe(NO3)3 mL | KSCN mL | H2O mL |
| --- | --- | --- | --- |
| 1 | 5 | 2 | 3 |
| 2 | 5 | 3 | 2 |
| 3 | 5 | 4 | 1 |
| 4 | 5 | 5 | none |

5.	Use a stir rod to mix the contents of each test tube thoroughly. Note: Be sure to rinse and dry the stir rod after you stir each test tube.

6.	Use the thermometer to measure the temperature of one of the solutions. Record the temperature in the Lab Report section.

### Prepare the Standard Thiocyanoiron Solution

To prepare the standard FeSCN2+ solution, add a large concentration of Fe3+ ions to a relatively small concentration of SCN- ions. The concentration of Fe3+ ions is so large that nearly all the SCN- ions will be used up. Therefore, for every mole of SCN- ions reacted, one mole of FeSCN2+ is produced. The concentration of the standard thiocyanoiron solution will be the same as the initial concentration of the SCN- ions.

•	For this part you need a test tube, a label, a pipette, 18 mL of 0.200 M Fe(NO3)3, 2 mL of 0.002 M KSCN, and a stir rod.

1.	Put the label on the test tube and mark it #5

2.	Use the pipette to put 18 mL of 0.200 M Fe(NO3)3 into the test tube.

3.	Add 2 mL of 0.002 M KSCN to the same test tube and use the stir rod to mix thoroughly.

## PART III: Data Recording - DataStudio

•	(Note: See the appendix for data recording using ScienceWorkshop.)

1.	Empty the water from the cuvette used during calibration. Using the solution in Test Tube #1, rinse the cuvette twice with approximately 1 mL amounts of solution from the test tube, and then fill the cuvette with solution. Cap the cuvette.

2.	Wipe the outside of the cuvette with a tissue and place the cuvette in the Colorimeter. Close the lid.

3. 	Arrange the Table display so you can see it clearly.

4.	When everything is ready, press the ‘Start/Stop’ button () on the Colorimeter and then start recording data.

•	The ‘Start’ button changes to a ‘Keep’ button (). The Table display shows a column for Absorbance and a column for the numbers of each test tube.

5.	When the Absorbance value stabilizes, click ‘Keep’ to record the Absorbance value for the sample from Test Tube #1.

6.	Press the ‘Start/Stop’ button to stop the Colorimeter. Remove the cuvette from the Colorimeter and empty the cuvette. Rinse the cuvette carefully with distilled water. Empty the water from the cuvette.

7.	Using the solution in Test Tube #2, rinse the cuvette twice with approximately 1 mL amounts and then fill the cuvette with solution from Test Tube #2. Cap the cuvette. Wipe the outside with a tissue and place the cuvette in the Colorimeter. Close the Colorimeter lid.

8.	Press the ‘Start/Stop’ button to start the Colorimeter. When the Absorbance value stabilizes, click ‘Keep’ to record the new Absorbance value. Press the ‘Start/Stop’ button again to stop the Colorimeter.

9.	Repeat the process to find the absorbance of the samples from Test Tubes #3, #4, and #5.

10.	After you record the Absorbance for the last solution, stop recording data. Press the ‘Start/Stop’ button to stop the Colorimeter. Rinse the cuvette with distilled water.

11.	Dispose of the solutions as directed.

## Analyzing the Data – DataStudio and ScienceWorkshop

1.	Record the absorbance data from the Table display into the Lab Report section.

2.	Calculate the initial concentration of Fe3+. The initial concentration, [Fe3+]i, is the amount of iron nitrate, Fe(NO3)3, in each test tube compared to the total volume in the test tube, multiplied by the concentration of the Fe(NO3)3.

[Fe3+]i =  Fe(NO3)3mLtotal mL´(0.002 M)

•	Note: The initial concentration is the same for all four test tubes.

[SCN−]i = KSCN mLtotal mL´(0.002 M)3.	Calculate the initial concentration of SCN- for each of the four samples. The initial concentration, [SCN-], is the amount of potassium thiocyanate, KSCN, in each test tube compared to the total volume in the test tube, multiplied by the concentration of the KSCN.

For example, in Test Tube #1 the amount of KSCN is 2 ml and the total volume is 10 mL. The concentration of KSCN is 0.002 M.

[SCN−]i = 2 mL10 mL´(0.002 M)= 0.004 M[FeSCN2+]eq =  AeqAstd´[FeSCN2+]std4.	Calculate the equilibrium concentration of FeSCN2+ for each of the four samples. The equilibrium concentration is the ratio of the equilibrium absorbance to the standard absorbance multiplied by the concentration of the FeSCN2+ in the standard solution.

FeSCN2+std=2 ml20 ml´0.002=0.0002 M•	Note: Remember that the concentration of the FeSCN2+ in the standard solution is the same as the concentration of the SCN- in the standard solution. Therefore, the concentration of the FeSCN2+ standard is as follows:

Fe3+eq=Fe3+i−FeSCN2+eq5.	Calculate the equilibrium concentration of Fe3+ for each of the four samples. The equilibrium concentration is the initial concentration of Fe3+ minus the equilibrium concentration of FeSCN2+.

SCN−eq=SCN−i−FeSCN2+eq6.	Calculate the equilibrium concentration of SCN- for each of the four samples. The equilibrium concentration is the initial concentration of SCN- minus the equilibrium concentration of FeSCN2+.

Kc=FeSCN2+Fe3+SCN−7.	Calculate the equilibrium constant, Kc, for each of the four samples.

## Record your results in the Lab Report section.

# Lab Report - Activity C19: Determine the Equilibrium Constant, Kc, of a Reaction

## What Do You Think?

In some chemical reactions the amounts of products formed can be calculated when the amounts of reactants are known, assuming that the reaction goes to completion so that one of the reactants is used up. However, many reactions to not go to completion. Instead, they reach an equilibrium state in which both reactants and products are present. The ratio of the concentrations of reactants to products at a given temperature is the equilibrium constant, Kc. The equilibrium constant allows the calculation of amounts of substances present at equilibrium. How can a Colorimeter be used to determine the equilibrium constant of a chemical reaction?

## Data Table

| Test Tube | #1 | #2 | #3 | #4 |
| --- | --- | --- | --- | --- |
| Absorbance |  |  |  |  |

| Absorbance of standard (Test Tube #5) |  |
| --- | --- |
| Temperature |  |

| Test Tube | #1 | #2 | #3 | #4 |
| --- | --- | --- | --- | --- |
| [Fe3+]i |  |  |  |  |
| [SCN-]i |  |  |  |  |
| [FeSCN2+]eq |  |  |  |  |
| [Fe3+]eq |  |  |  |  |
| [SCN-]eq |  |  |  |  |
| Kc |  |  |  |  |

| Average Kc value |  |
| --- | --- |

## Questions

1.	What is the average value of the equilibrium constant, Kc, for the four samples?

2.	How constant is the value of Kc for the four samples?

## Appendix: Set Up ScienceWorkshop

Create a ScienceWorkshop file to measure and display absorbance.

### Set Up the Sensors

1.	In the Experiment Setup window, click and drag the analog sensor plug to Channel A.

2.	Select ‘Colorimeter’ from the list of sensors. Click ‘OK’ to return to the Experiment Setup window.

### Set Up the Sampling Options

1.	In the Experiment Setup window, click the ‘Sampling Options…’ button (or select it from the Experiment menu).

2.	In the Sampling Options window, click the check box in front of ‘Keyboard’. Enter ‘Test Tube Number’ as the Parameter. Leave ‘Units’ blank. Click ‘OK’ to return to the Experiment Setup window.

### Set Up the Displays

1.	In the Experiment Setup window, click and drag the Table display icon to the Colorimeter icon.

2.	In the Table display, add a column for ‘Test Tube Number’. Click the Add-a-Column menu button () and select ‘Test Tube Number’ from the list.

3.	In the Experiment Setup window, click and drag the Digits display icon to the Colorimeter icon.

## Record Data

1.	Empty the water from the cuvette used during calibration. Using the solution in Test Tube #1, rinse the cuvette twice with approximately 1 mL amounts of solution from the test tube, and then fill the cuvette with solution. Cap the cuvette.

2.	Wipe the outside of the cuvette with a tissue and place the cuvette in the Colorimeter. Close the lid.

3.	When you are ready, press the ‘Start/Stop’ button () to start the Colorimeter. Click ‘REC’ to start recording data.

•	The ‘Keyboard Sampling’ window will open. Arrange the windows so you can see the Digits display.

4.	When the readings for Absorbance stabilize, type ‘1’ in the Keyboard Sampling window and click ‘Enter’ to record the Absorbance for the first sample. Press the ‘Start/Stop’ button to stop the Colorimeter.

5.	Remove the cuvette from the Colorimeter and empty the cuvette. Rinse the cuvette carefully with distilled water. Empty the water from the cuvette.

6.	Using the solution in Test Tube #2, rinse the cuvette twice with approximately 1 mL amounts and then fill the cuvette with solution from Test Tube #2. Cap the cuvette. Wipe the outside with a tissue and place the cuvette in the Colorimeter. Close the Colorimeter lid.

7.	Press the ‘Start/Stop’ button to start the Colorimeter. When the Absorbance value stabilizes, type ‘2’ in the Keyboard Sampling window and click ‘Enter’ to record the new Absorbance value. Press the ‘Start/Stop’ button again to stop the Colorimeter.

8.	Repeat the process to find the absorbance of the samples from Test Tubes #3, #4, and #5.

9.	After you record the Absorbance for the last solution, click ‘Stop Sampling’ () to stop recording data. The Keyboard Sampling window will automatically close. Press the ‘Start/Stop’ button to stop the Colorimeter. Rinse the cuvette with distilled water.

10.	Dispose of the solutions as directed.

## Analyze the Data

See the ‘Analyzing the Data’ section.

# Record your results in the Lab Report section.