Eleven Cent Battery

| Equipment Needed | Consumables |
| --- | --- |
| • ScienceWorkshop® Interface | • heavy watercolor paper or thick paper towel |
| • Voltage Sensor | • 1 dime and 1 penny |
|  | • salt water |
|  | • eraser |

Purpose

The purpose of this activity is to use the Voltage Sensor to measure the voltage produced by a coin “battery” and then compare this to the voltage of an actual D-cell battery.

Procedure

|  | Turn on the interface box and the computer. Plug a Voltage Sensor into analog channel A of the interface box. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Open the Experiment Library template file: |  |  |  |  |  |
|  |  | ScienceWorkshop | DataStudio |  |  |  |
|  |  | MSS30 | MSS30 Eleven Cent Battery.ds |  |  |  |
|  | Make sure you have the correct, color-coded alligator clips on the ends of the Voltage Sensor wire leads. These clips will connect to the free ends of the copper wire. |  |  |  |  |  |

|  | Clean each of the coins by rubbing them with an eraser. They should be shiny on all surfaces. Wash them with soap and water, then rinse and dry them. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Cut the watercolor paper (or heavy paper towel) into a round circle slightly larger than a penny. If your paper towel is thin, cut two circles. Soak these pieces in salt water. |  |  |  |  |  |
|  | Connect one of the alligator clips of the Voltage Sensor to the penny and the other to the dime. Place the paper towel circle(s) between the coins. |  |  |  |  |  |
|  | Begin recording data. Record for 60 seconds, and then stop recording data. |  |  |  |  |  |
|  |  | If you get very low voltage readings, re-soak the paper circles in the salt water solution. |  |  |  |  |
|  | Re-scale the graph, and investigate interesting areas of your data with the Smart Cursor or Smart Tool. |  |  |  |  |  |
|  |  |  | Compare the voltage produced by the battery you made from coins and salt water-soaked paper to the voltage produced by an actual D-cell |  |  |  |
|  |  |  | What changes could you make in your 11-cent battery that would affect the amount of voltage it produces? Why is salt water used instead of tap water? |  |  |  |
|  |  |  | Be sure to follow your teacher’s instructions for putting away your equipment and shutting down the computer. If you are saving your data on a disk, remember to give it a new file name. |  |  |  |
|  | Optional: |  | While still connected to the Voltage Sensor, place the coins in the saltwater solution. Did the voltage increase, decrease or stay the same? |  |  |  |