# Activity P55: Transistor Lab 1 – The NPN Transistor as a Digital Switch (Power Output, Voltage Sensor)

| Concept | #### DataStudio | #### ScienceWorkshop (Mac) | #### ScienceWorkshop (Win) |
| --- | --- | --- | --- |
| Semiconductors | P55 Digital Switch.DS | (See end of activity) | (See end of activity) |

| ## Equipment Needed | ## Qty | ## From AC/DC Electronics Lab* | ## Qty |
| --- | --- | --- | --- |
| Voltage Sensor (CI-6503) | 1 | Light-emitting diode (LED), red | 1 |
| Alligator Clip Adapters (SE-9756) | 2 | Resistor, 330 ohm (Ω) | 1 |
| Patch Cord (SE-9750) | 4 | Resistor, 22 kilo-ohm (Ω) | 1 |
| Power Supply, 5 V DC (SE-9720) | 1 | Transistor, 2N3904 | 1 |
|  |  | Wire Lead, 5 inch | 2 |

## (* The AC/DC Electronics Lab is EM-8656)

## What Do You Think?

Using your library or the Internet, explore the following: What key aspects of the transistor aided in revitalizing Japan’s economy after WWII?

Take time to answer the ‘What Do You Think?’ question(s) in the Lab Report section.

The purpose of this activity is to investigate how the npn transistor operates as a digital switch.

## Background

The transistor is the essential ingredient of every electronic circuit, from the simplest amplifier or oscillator to the most elaborate digital computer. Integrated circuits (IC’s) which have largely replaced circuits constructed from individual transistors, are actually arrays of transistors and other components built from a single wafer-thin piece or “chip” of semiconductor material.

The transistor is a semiconductor device that includes two p-n junctions in a sandwich configuration which may be either p-n-p or, as in this activity, n-p-n. The three regions are usually called the emitter, base, and collector.

In a transistor circuit, the current to the base controls the current through the collector “loop”.

The collector voltage can be considerably larger than the base voltage. Therefore, the power dissipated by the resistor may be much larger than the power supplied to the base by its voltage source. The device functions as a power amplifier (as compared to a step-up transformer, for example, which is a voltage amplifier but not a power amplifier). The output signal can have more power in it than the input signal. The extra power comes from an external source (the power supply). A transistor circuit can amplify current or voltage. The circuit can be a constant current source or a constant voltage source.

A transistor circuit can serve as a ‘digitial’ electric switch. In a mechanical electric switch, a small amount of power is required to ‘switch on’ an electrical device (e.g., a motor) that can deliver a large amount of power. In a ‘digital’ transistor circuit, a small amount of power supplied to the base is used to “switch on” a much larger amount of power from the collector.

Here is some general information. A transistor is a three-terminal device. Voltage at a transistor terminal relative to ground is indicated by a single subscript. For example, VC is the collector voltage. Voltage between two terminals is indicated by a double subscript: VBE is the base-to-emitter voltage drop, for instance. If the same letter is repeated, it means a power-supply voltage: VCC is the positive power-supply voltage associated with the collector.

A typical npn transistor follows these rules:

1.	The collector must be more positive than the emitter.

2.	The base-to-emitter and base-to-collector circuits behave like diodes. The base-emitter diode is normally conducting if the base is more positive than the emitter by 0.6 to 0.8 volts (the typical forward “turn on” voltage for a diode). The base-collector diode is reverse-biased.

3.	The transistor has maximum values of IC, IB, and VCE and other limits such as power dissipation (ICVCE) and temperature.

- If rules 1 – 3 are obeyed, the current gain (or amplification) is the ratio of the collector current, IC, to the base current, IB. A small current flowing into the base controls a much larger current flowing into the collector. The ratio, called “beta”, is typically around 100.

| ## SAFETY REMINDER - Follow all safety instructions. |  |
| --- | --- |

## For You To Do

Use the ‘Output’ feature of the ScienceWorkshop interface to supply an AC voltage to the base of the npn transistor. Use the DC power supply to supply a voltage to the collector of the transistor. Use one Voltage Sensor to measure the voltage drop (potential difference) across a resistor in series with the power supply and the collector of the transistor.

Use DataStudio or ScienceWorkshop to record and display the ‘Output Voltage’ to the base of the transistor (Vbase) and the voltage drop across the resistor in series with the collector (Vcollector). Find the value of ‘Vbase’ (voltage across the base) that causes the value of ‘Vcollector’ to increase from zero. In other words, determine the voltage at which the transistor ‘switches on’.

## PART I: Computer Setup

1.	Connect the ScienceWorkshop interface to the computer, turn on the interface, and turn on the computer.

2.	Connect the Voltage Sensor to Analog Channel A.

3.	Connect two banana plug patch cords into the ‘OUTPUT’ ports on the interface.

4.	Open the document titled as shown:

| #### DataStudio | #### ScienceWorkshop (Mac) | #### ScienceWorkshop (Win) |
| --- | --- | --- |
| P55 Digital Switch.DS | (See end of activity) | (See end of activity) |

•	The DataStudio document has a Graph display and a Workbook display. Read the instructions in the Workbook.

•	See the pages at the end of this activity for information about modifying a ScienceWorkshop file.

•	The Signal Generator is set to output a 1.6 volt ‘Sine Wave’ at 1 Hz. The output is set to start and stop automatically when you start and stop measuring data.

•	Data recording is set at 200 Hz with a Start Condition of ‘Output Voltage’ going above 0.01 V and a Stop Condition of Time equal 1 second (about 200 samples).

## PART II: Sensor Calibration and Equipment Setup

•	You do not need to calibrate the Voltage Sensor.

1.	Insert the 2N3904 transistor into the socket on the AC/DC Electronics Lab circuit board. The transistor has a half-cylinder shape with one flat side. The socket has three holes labeled “E” (emitter), “B” (base) and “C” (collector). When held so the flat side of the transistor faces you and the wire leads point down, the left lead is the emitter, the middle lead is the base, and the right lead is the collector.

2.	Connect the 22-kΩ resistor (red, red, orange) vertically between the component springs at the left edge of the component area.

3.	Connect the 330-Ω resistor (orange, orange, black) horizontally between the component springs to the left of top banana jack.

4.	Carefully bend the wire leads of the red light-emitting diode (LED) so it can be mounted between component springs. Connect the LED between the component springs to the left of the 330-Ω resistor. Arrange the LED so its cathode (short lead) is to the left (away from the resistor).

5.	Connect a wire lead from the component spring at the base terminal of the transistor to the component spring at the top of the 22-kΩ resistor.

6.	Connect another wire lead from the component spring at the collector terminal of the transistor to the component spring at the left end of the LED.

7.	Connect a patch cord from the positive (+) terminal of the DC power supply to the top input jack on the edge of the circuit board.

8.	Use an alligator clip adapter to connect another patch cord from the negative (-) terminal of the DC power supply to the component spring of the emitter terminal of the transistor

9.	Use an alligator clip adapter to connect the patch cord from the positive () output port of the ScienceWorkshop interface to the component spring below the 22 kΩ resistor on the circuit board.

10.	Connect a black banana plug patch cord from the ground () output port of the interface to the negative (-) terminal of the DC power supply.

To GROUND on DC Power Supply

To Channel A

To +5 volt on DC Power Supply

To ‘OUTPUT’ on Interface

11.	Put alligator clips on the banana plugs of the Voltage Sensor. Connect the red lead of the sensor to the component spring at the right end of the 330-Ω resistor and the black lead to the left end of the resistor.

## PART III: Data Recording

1.	Turn on the DC power supply and adjust its voltage output to exactly +5 Volts.

2.	Start recording data. (Click ‘Start’ in DataStudio or ‘REC’ in ScienceWorkshop.)

•	Observe the behavior of the LED. Write a description of what you observe.

•	Recording will stop automatically at 1 second.

3.	Turn off the DC power supply.

## Analyzing the Data

•	Optional: Save your data. If a printer is available, print the Graph display.

- Remember, the Channel A voltage is Vcollector and the ‘Output Voltage’ (from the interface) is Vbase.

- Set up your Graph display so it fits the data.

- Hint: In DataStudio, click the ‘Scale to Fit’ button in the Graph toolbar. In ScienceWorkshop, click the ‘Autoscales button () to rescale the Graph.

- Use the Graph’s built-in analysis tool to measure the voltage to the base (Vbase) when the voltage to the collector (Vcollector) first begins to increase above zero.

- Hint: In DataStudio, click the ‘Smart Tool’. The Smart Tool is a large cross hair with an ordered pair of numbers that show the X-value and Y-value of its position. Place the Smart Tool at the point on Vbase (‘Output Voltage’) that matches the point where Vcollector (Voltage, ChA) first increases above zero.

- Hint: In ScienceWorkshop, click the ‘Smart Cursor’ button. The cursor changes to a cross hair when you move it into the display area. The X-coordinate of the cursor/cross-hair is displayed under the horizontal axis. The Y-coordinate of the cursor/cross-hair is displayed next to the vertical axis. Put the cursor at the point on the plot of Vcollector (Channel A) where the voltage first begins to increase above zero. Hold down the Shift key. While holding the Shift key, move the cursor/cross-hair vertically along the dashed line until you reach the point on the plot of Vbase (Output Voltage) that corresponds to the same point on the plot of Vcollector.

- Record the Y-coordinate of that point on the plot of Vbase.

Voltage = _________ (V)

# Record your results in the Lab Report section.

# Lab Report - Activity P55: Transistor Lab 1 – The NPN Transistor as a Digital Switch

## What Do You Think?

Using your library or the Internet, explore the following: What key aspects of the transistor aided in revitalizing Japan’s economy after WWII?

## Data

Voltage =             V

## Questions

- What is the behavior of the LED when the circuit is active?

- How does the general shape of the plot for the Vbase compare to the plot of Vcollector for the transistor?

- What is the voltage on the Vbase plot when the LED turns on (that is, when the Vcollector voltage begins to rise above zero – the ‘switch on’ voltage)?

- What is the relationship between the behavior of the LED and the point on the plot of Vcollector when the voltage begins to rise above zero?

## Modify an existing ScienceWorkshop file.

### Open the ScienceWorkshop File

Open the file titled as shown:

| ScienceWorkshop (Mac) | ScienceWorkshop (Win) |
| --- | --- |
| P48 Transistor Lab 1 | P48_TRN1.SWS |

This activity uses the ‘Output’ feature of the ScienceWorkshop 750 interface to provide the output voltage. Remove the Power Amplifier in the Experiment Setup window.

### Remove the Power Amplifier Icon

In the Experiment Setup window, click the Power Amplifier icon and press <delete> on the keyboard.

Result: A warning window opens. Click ‘OK’ to return to the setup window.

### Modify the Signal Generator

Set the Signal Generator to output a 1.6 volt ‘Sine Wave’ at 1 Hz.

### Check the Graph Display

The Graph display should show ‘Output Voltage’ and ‘A’. If not, use the Input Menu to select ‘Output Voltage’ for the top plot and ‘A’ for the bottom plot.